Maryland-National Capital Region Area Contingency Plan (MNCR-ACP). Air Operations and UAS Support Annex K May 2021.3

Record of Changes

| Change Number | Change Description | Part Number | Change Date | Name |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

Table of Contents

1000 Introduction	1

2000 Aircraft	1

2100 Coast Guard Aircraft	1

2200 Civil Air Patrol (CAP) Photo Downlink	1

2300 Unmanned Aerial Systems(UAS)	1

3000 Air Operations	1

3100 Aerial Dispersant Surveillance	1

3110 Spotter Aircraft	1

3120 Monitor Aircraft	1

3130 Observation Aircraft	2

3200 Aerial Dispersant Application	2

4000 Flight Restrictions	3

4100 Procedures for Temporary Flight Restrictions	3

4200 Procedures for Temporary Flight Restrictions	3

5000 Air Support	4

5100 Airports/Helibases	4

5200 Airports	6

5300 Helispots / Overflight Information	7

5400 List of Certified Helos/Aircraft Providers	11

5500 Fuel/Maintenance Sources	11

5600 Air Traffic Control Procedures	11

List of Tables

Table 1 Aerial Dispersant Capability	2

Table 2 Maryland Airports	4

Table 3 Helispots / Overflight Information	7

Table 4 Mass Rescue Operation Supplemental Checksheet ……………………………………..11

# 1000 Introduction

This section provides guidance on air operations and support resources available for pollution response and emergency incidents within the Maryland-NCR Area. It outlines procedures for requesting Coast Guard aircraft, utilizing Civil Air Patrol assets, and deploying unmanned aerial systems. The section details aerial surveillance and dispersant application capabilities, including commercial providers and operational roles such as spotter, monitor, and observation aircraft. It also covers flight restrictions, air support functions, airport and helibase locations, helispots, certified aircraft providers, fuel and maintenance sources, and air traffic control procedures. This information ensures effective coordination, safety, and operational readiness during air-based response activities.

# 2000 Aircraft

## 2100 Coast Guard Aircraft

The East Coast Guard District Operations Center is the point of contact to request the use of Coast Guard aircraft. Their phone number is: (757) 398-6231.

## 2200 Civil Air Patrol (CAP) Photo Downlink

The CAP is the Air Force Auxiliary. They operate a single frame video downlink system from their airplanes. Typical cost is $120 per hour for two planes, including all crew and equipment. They can be activated by the USCG through HQ CAP-USAF and reimbursed through a Pollution Funding Removal Authorization (PFRA) for Oil & HAZMAT spills or a Military Interagency Procurement Request (MIPR) for other missions.

HQ CAP and HQ CAP-USAF:					(334) 953-4225

https://www.cap.gov/							(334) 953-4223

(334) 953-4232

(888) 211-1812

## 2300 Unmanned Aerial Systems(UAS)

Under development.

# 3000 Air Operations

## 3100 Aerial Dispersant Surveillance

Specific to dispersant applications, surveillance is responsible for directing and coordinating air operations missions to apply dispersants and conduct oil spill tracking, observation, and remote sensing.

### 3110 Spotter Aircraft

The Spotter Aircraft Position or “Spotter” is physically located in an aircraft. The Spotter is a person who “spots” or controls, guides, or lines up the sprayer aircraft or vessels over the spill target. Because a dispersant application can be made by both vessels and aircraft, the Spotter would maintain tactical control over both types of delivery systems. The Spotter is in charge of the dispersant operation on scene. Because dispersant operations can be executed in multiple geographic areas due to the spreading and breakup of the slick, multiple spotter aircraft may be needed (one for each spray aircraft).

### 3120 Monitor Aircraft

The monitor aircraft or vessel or the "monitor" is primarily responsible for monitoring the effectiveness of the dispersant operation through aerial observation in aircraft and through the use of fluorometers on board vessels to sample the dispersed oil. Effectiveness monitoring is concerned primarily with determining whether the dispersant was properly applied and how the dispersant is affecting the oil.

### 3130 Observation Aircraft

The observation aircraft or vessels "observers" are platforms and persons specifically assigned to observe the dispersant operation. Their observer status should be authorized by the Unified Command on the basis of their position as a stakeholder in the outcome of the operation.

Observers might include corporate officials, agency representatives, political officials, scientists, trustees, interest group representatives, and so forth.

Refer to the Incident Management Handbook for Oil Spill Aerial Observer information.

## 3200 Aerial Dispersant Application

Below is a list of commercial providers of aerial dispersant application.

Table 1 Aerial Dispersant Capability

| Name of Supplier/ Location | Contact Information | Equipment |
| --- | --- | --- |
| Air Response, Inc. Mesa, AZ | Richard Packard (480) 844-0800 | 1 DC-4 airplane equipped with 2,000 capacity in-line spray system |
| Clean Harbors Environmental Services https://www.cleanharbors.com/ | Edison, NJ (800) 645-8265 | COREXIT 9527: 1,375 G in 55G drums in trailer 1 workboat spray system1 220G helo bucket |
| Delaware Bay & River Co-Op Lewes, DE | Gene Johnson (302) 645-7861 | COREXIT 9527: 1,650G in 55G drums 1 VOSS spray system |
|  |  | 1 TC3 helo bucket |
| Airborne Support, Inc. Houma, LA | Howard Barker Brad Barker (985) 851-6391 | COREXIT 9527G DC-4 plane W/24,000G cap DC-3 plane w/1,200G and 1,000G cap Twin engine spotter plane Assoc’d loading pumps |
| Farwest/Biegert Aviation, Inc. Chandler, AZ | Jim Jefferies David Berry (520) 796-2400 | 2 ADDS-PACK systems Ancillary pumping equipment |
| Clean Caribbean Co-Op Port Everglades, FL | Skip Przelomski (954) 983-9880 | COREXIT 9527 5,000G in bulk tank; 55G drums COREXIT 9500 15,840G in 55G drums 1 ADDS-PACK Unit 2 VOSS spray systems 2 helo spray buckets |
| https://www.cleancaribbean.org/ |  |  |
| Emergency Aerial Dispersants Consortium (Agricultural spraying aircraft) | (207) 665-2362 | AT-802 Aircraft - can fly up to 200 mi- offshore -810 gal. Capacity - COREXIT 9500 |
|  | Email: vervision@aol.com |  |
| MSRC |  |  |
| Clean Seas, Carpinteria, CA | (800) 982-1948 | COREXIT 9527 11,000G in 55G drums 2 90G helo buckets |
|  | (805) 684-3838 |  |
| NALCO/Exxon Energy Chemicals LP Sugarland, TX | Steve Sears- Material Control Manager (281) 263-7404 Cell Phone (281) 782-9780 Main Office (281) 263-7000 | COREXIT 9527 200,000G in 55G drums COREXIT 9500 stored 55G drums |
| NRC Miami, FL https://nrcc.com/ | Bob Grim (305) 379-1625 | COREXIT 9527 5,000G in 55G drums |
| X Products and Services, Inc. Colorado Springs, CO | John Kuipers (719) 576-8047 | SX-100 4,840G in 55G drums |
| CISPRI-Cook Inlet Spill Response Prevention and Response Inc. North Kenai, AK | Victoria Askin (907) 776-7406 Main Office | COREXIT 9527 11,275G in 55G drums 2 helo buckets |

# 4000 Flight Restrictions

## 4100 Procedures for Temporary Flight Restrictions

Due to the presence of three major and several regional airports in this area, it is necessary to be aware of possible interference with airspace even for a ‘routine over-flight’. In all cases, the Federal Aviation Administration (FAA) and/or nearest airport that could be affected should be contacted.

NOTAMS or similar advisories can be posted/broadcasted by the FAA to alert aviators of possible environmental hazards. Likewise, response personnel and media engaged in assessment or follow-up surveillance of a spill site, need to be fully aware of FAA or DOD controlled airspace and any hazards or restrictions that may exist.

Links for more info:

- Washington, DC Area Airports: https://www.mwaa.com

- BWI Airport: https://www.bwiairport.com/

- FAA Air Traffic Control Command Center: https://www.fly.faa.gov/flyfaa/usmap.jsp

- Temporary Flight Restrictions & Daily Advisories: https://pilotweb.nas.faa.gov/

## 4200 Procedures for Temporary Flight Restrictions

As a result of the events of September 11, 2001, the FAA and Department of Defense have intensified their efforts to monitor and restrict air traffic within the Baltimore and Metropolitan Washington, DC air corridors. Several permanent area flight restriction zones have been expanded or established, including the Washington, DC Metropolitan Area Flight Restricted Zone (FRZ) and the Air Defense Identification Zone since September 11, 2001. In addition, additional temporary flight restrictions may be imposed during Special Security Events or Incidents of Regional or National Significance. Emergency responders should monitor appropriate aircraft warning frequencies and Notices to Airmen (NOTAMs) for the latest information.

For further information, please visit the following websites:

FAA’s Pilot Web Site:

https://pilotweb.nas.faa.gov/distribution/atcscc.html

A Pilot’s Guide to Understanding Restrictions in Today’s National Airspace System: https://www.faa.gov/pilots/safety/notams_tfr/media/tfrweb.pdf

In addition, there are flight space restrictions elsewhere within the Area Committee zone, please consult FAA’s Pilot Website.

# 5000 Air Support

The Air Support Group Supervisor is responsible for supporting and managing Helibase and Helispot operations and maintaining liaison with Fixed- winged air bases. This includes:

- Providing fuel and other supplies

- Providing maintenance and repair of helicopters

- Keeping records of helicopter activity

- Providing enforcement of safety regulations.

These major functions are performed at Helibases and Helispots. Helicopters during landing, takeoff, and while grounded, are under the control of the Air Support Group's Helibase or Helispot managers. The Air Support Group Supervisor reports to the Air Operations Branch Director.

## 5100 Airports/Helibases

The following lists the airports of the COTP zone by county. Imbedded in the listings are further hyperlinks to homepages for the airports, which contain additional information about each field including size, contact info, and the availability of fuel.

Table 2 Maryland Airports

| County | Airport(s) / Landing Area(s) | Phone |
| --- | --- | --- |
| Anne Arundel County | Baltimore Washington Int'l | 410-859-7111 |
|  | Lee Airport | 410-956-1280 |
| Baltimore County | Martin State Airport | 410-682-8810 |
| Caroline County | Ridgely Airport | 410-923-3660 |
| Charles County | Maryland Airport | 301-283-6202 301-375-7337/7606 |
| Cecil County | Claremont Airport | 410-398-0234 |
| Dorchester County | Cambridge - Dorchester Regional Airport | 410-228-4571 |
| Harford County | Harford County Airport | 410-836-2828 |
|  | Fallston Airport | 410-877-9889 |
| Prince Georges County | Andrews Air Force Base | 301-981-3411/1110 |
|  | College Park | 301-864-5844 |
|  | Hyde Field/ Washington Executive Airpark | 301-297-7290 |
|  | Potomac Airfield | 301-248-5720 |
|  | Freeway Airport | 301-390-6424 |
| Queen Anne’s County | Kentmorr Airport | 410-643-1785 |
|  | Bay Bridge Airport | 410-643-4364 |
| Somerset County | Crisfield - Somerset County Airport | 410-968-3062 |
| St. Mary’s County | St. Mary’s County Regional Airport | 301-373-2101 |
| Talbot County | Easton/ Newman Field Airport | 410-770-8055 |
| Washington D.C. | Ronald Reagan Int'l Airport | 703-417-8000 |
| Wicomico County | Bennett Airport (private grass runway) | 410-859-7137 410-859-7065 |
|  | Bayland Aviation – Refueling At Salisbury-Ocean City Wicomico Regional http://baylandaviation.com/index.html | 410-749-0323 |
|  | Salisbury-Ocean City Wicomico Regional Airport (capable of landing up to 757s) | 410-548-4827 |
| Worcester County | Ocean City Municipal Airport | 410-213-2471 |

## 5200 Airports

The following airports located in the Maryland-NCR Area that have hard surfaced runways of at least 5000 feet long with 24 hour operation.

Figure 1 Maryland’s Public Airports

Baltimore-Washington International (BWI) 				(410) 859-7040

Baltimore, MD

Lat 39o-30.5'N Long 076o-40.0'W, Length 9500 ft.

https://www.airnav.com/airport/KBWI

Salisbury-Wicomico County Regional				(410) 548-4827

Salisbury, MD

Lat 38o-20.5'N Long 075o-30.3'W, Length 5000 ft.

Martin State								(410) 682-8800

Baltimore, MD

Lat 39o-19.7'N Long 076o-25.0'W, Length 7000 ft.

Patuxent River Naval Air Station (U.S. Navy)			(301) 342-3836

Patuxent River, MD

Lat 38o-17.3'N Long 076o-25.0'W, Length 11800 ft.

https://www.airnav.com/airport/KNHK

Phillips Army Air Field (U.S. Army)					(410) 278-4902

Aberdeen, MD

Lat 39o-28.0'N Long 076o-10.0'W, Length 8000 ft.

https://www.airnav.com/airport/KAPG

Dover Air Force Base (U.S. Air Force)				(302) 677-3000

Dover, DE

Lat 39o-09.0'N Long 076o-31.7'W, Length 9600 ft.

https://www.airnav.com/airport/DOV

Ronald Reagan National Airport					(703) 417-8000

Washington, D.C.

Lat 38o-50.5'N Long 077o-02.5'W, Length 6800 ft.

https://www.airnav.com/airport/DCA

https://www.fltplan.com/AirportInformation/KDCA.htm

Andrews Air Force Base (U.S. Air Force)				(301) 981-1110

Camp Spring, MD

Lat 38o-49.0'N Long 076o-52.0'W, Length 9700 ft.

https://www.airnav.com/airport/KADW

Dulles International (IAD)						(703) 572-2700

Washington, D.C.

Lat 38o-57.0'N Long 077o-27.0'W, Length 11500 ft.

https://www.airnav.com/airport/IAD

## 5300 Helispots / Overflight Information

Table 3 Helispots / Overflight Information

| City | Facility Name | ARP Latitude | ARP Longitude |
| --- | --- | --- | --- |
| ABERDEEN | RITE AID DISTRIBUTION CENTER | 39-27-12.3900N | 076-12-33.8300W |
| ANNAPOLIS | ANNE ARUNDEL MEDICAL CENTER | 38-59-19.0000N | 076-31-54.0000W |
| ARBUTUS | SECURITY FORD | 39-14-45.3840N | 076-40-33.8880W |
| BALTIMORE | MONTEBELLO FILTRATION PLANT | 39-20-10.0000N | 076-35-12.0000W |
| BALTIMORE | GREEN TERRACE | 39-24-09.3850N | 076-30-17.8710W |
| BALTIMORE | UNIVERSITY OF MARYLAND SHOCK TRAUMA CENTER | 39-17-19.3850N | 076-37-32.8830W |
| BALTIMORE | SINAI HOSPITAL | 39-21-07.3830N | 076-39-41.8910W |
| BALTIMORE | FRANKLIN SQUARE HOSPITAL CENTER | 39-21-02.0000N | 076-28-35.0000W |
| BALTIMORE | SINAI II | 39-21-28.0000N | 076-39-45.0000W |
| BALTIMORE | JOHN HOPKINS - BAYVIEW | 39-17-28.9000N | 076-32-46.5500W |
| BALTIMORE | MARRIOTT PARKING GARAGE "ROOFTOP" | 39-16-59.7000N | 076-36-07.0800W |
| BALTIMORE | ST. AGNES HEALTH CARE | 39-16-14.8000N | 076-40-23.0800W |
| BALTIMORE | PIER 7 | 39-16-20.0000N | 076-34-18.0000W |
| BALTIMORE | THE JOHNS HOPKINS HOSPITAL | 39-17-50.3850N | 076-35-35.8790W |
| BALTIMORE | BALTIMORE POLICE DEPARTMENT | 39-17-25.3850N | 076-36-26.8800W |
| BEL AIR | UPPER CHESAPEAKE MEDICAL CENTER | 39-31-20.9900N | 076-19-35.9900W |
| BELTSVILLE | BELTSVILLE SHOP | 39-03-00.3930N | 076-53-25.9040W |
| BETHESDA | SUBURBAN | 38-59-50.3950N | 077-06-35.9220W |
| BROOKLANDVILLE | BROOKLANDVILLE | 39-25-00.0000N | 076-40-40.0000W |
| CAMBRIDGE | DORCHESTER GENERAL HOSPITAL | 38-34-18.4270N | 076-04-03.7830W |
| CHESAPEAKE CITY | CHESAPEAKE CITY | 39-31-05.0400N | 075-49-17.2200W |
| CHESTERTOWN | CHESTER RIVER MEDICAL CENTER | 39-19-04.3980N | 076-03-49.8090W |
| CHEVERLY | PRINCE GEORGE'S HOSPITAL CENTER | 38-55-49.0000N | 076-55-15.0000W |
| CLINTON | SOUTHERN MD HOSPITAL CENTER | 38-44-53.0000N | 076-52-38.0000W |
| COLUMBIA | HOWARD COUNTY GENERAL HOSPITAL | 39-12-52.3830N | 076-53-10.9090W |
| COLUMBIA | AEROSPACE TECH CENTER | 39-14-11.3830N | 076-49-33.9050W |
| CRISFIELD | MCCREADY MEMORIAL HOSPITAL | 37-58-00.0000N | 075-51-58.7570W |
| CUMBERLAND | WESTERN MARYLAND REGIONAL MEDICAL CENTER | 39-38-48.9000N | 078-43-55.9200W |
| EAGLE HARBOR | CHALK POINT GENERATING STA | 38-33-23.4360N | 076-41-33.8670W |
| EASTON | MEMORIAL HOSPITAL | 38-46-20.0000N | 076-04-19.0000W |
| EMMITSBURG | NAT'L EMERGENCY TRAINING CNTR | 39-41-06.0000N | 077-19-06.0000W |
| FORT DETRICK(FREDERICK) | FORT DETRICK HELIPAD | 39-26-11.3660N | 077-25-13.9470W |
| FORT RITCHIE | FORT RITCHIE | 39-42-00.3430N | 077-29-58.9620W |
| FORT WASHINGTON | FORT WASHINGTON MEDICAL CENTER | 38-43-40.2000N | 076-59-31.7500W |
| FREDERICK | GRIMES PROPERTIES | 39-24-30.3670N | 077-23-09.9460W |
| FREDERICK | FREDERICK MEMORIAL HOSPITAL | 39-25-18.0000N | 077-24-51.0000W |
| GAITHERSBURG | IBM | 39-09-12.3850N | 077-12-59.9350W |
| GLEN BURNIE | TAR COVE | 39-08-42.3930N | 076-30-04.8620W |
| GLEN BURNIE | BALTIMORE WASHINGTON MEDICAL CENTER | 39-08-15.3910N | 076-37-23.8780W |
| HAGERSTOWN | WASHINGTON COUNTY HOSPITAL | 39-38-14.3460N | 077-42-52.9740W |
| HAGERSTOWN | CRAIG COMPANY | 39-38-30.3450N | 077-44-58.9770W |
| HAGERSTOWN | ALLEGHENY POWER-HAGERSTOWN CORP CTR | 39-35-57.0000N | 077-45-50.0000W |
| HAVRE DE GRACE | GREGORY MAY | 39-31-56.0227N | 076-06-12.9839W |
| HUNT VALLEY | PHH | 39-29-39.0000N | 076-39-24.0000W |
| JACKSONVILLE | MRS BOZMAN | 39-32-58.3800N | 076-33-45.8830W |
| LA PLATA | CIVISTA MEDICAL CENTER | 38-31-42.6000N | 076-58-28.0000W |
| LAUREL | CITIZENS BANK HEADQUARTERS | 39-05-13.3900N | 076-53-49.9060W |
| LAUREL | LAUREL REGIONAL HOSPITAL | 39-05-15.3910N | 076-52-53.9040W |
| LAYTONSVILLE | FEDERAL SUPPORT CENTER | 39-11-31.3830N | 077-06-23.9270W |
| LEONARDTOWN | ST. MARY'S HOSPITAL EAST | 38-18-05.4000N | 076-38-12.6000W |
| LEONARDTOWN | ST. MARY'S HOSPITAL | 38-18-00.4500N | 076-38-13.8500W |
| LINTHICUM HEIGHTS | MARITIME INSTITUTE | 39-12-40.3860N | 076-40-18.8860W |
| LUSBY | CLNG COVE POINT | 38-23-30.4440N | 076-24-28.8200W |
| NEW CARROLLTON | METROPLEX | 38-56-46.4000N | 076-52-05.9000W |
| OAKLAND | GARRETT COUNTY MEMORIAL HOSPITAL | 39-24-47.3470N | 079-24-04.1600W |
| OCEAN CITY | HOOPERS | 38-18-41.4140N | 075-07-13.6660W |
| OCEAN CITY | 65TH STREET | 38-23-21.4100N | 075-04-19.6600W |
| OLNEY | NORWOOD | 39-07-38.0000N | 077-01-21.0000W |
| PRINCE FREDERICK | CALVERT MEMORIAL HOSPITAL | 38-33-39.0000N | 076-35-45.5000W |
| QUEENSTOWN | THE ASPEN INSTITUTE | 38-54-33.4100N | 076-07-10.8030W |
| RANDALLSTOWN | NORTHWEST HOSPITAL | 39-21-32.3800N | 076-46-53.9070W |
| ROCKVILLE | SHADY GROVE ADVENTIST HOSPITAL | 39-05-53.3890N | 077-11-48.9310W |
| SALISBURY | PENINSULA RGNL MEDICAL CENTER | 38-21-00.0000N | 075-35-00.0000W |
| SILVER SPRING | DOW JONES & CO. INC. | 39-02-47.3930N | 076-59-00.9140W |
| STEVENSON | BERG'S FIELD | 39-24-40.3810N | 076-42-58.9020W |
| TAKOMA PARK | WASHINGTON ADVENTIST HOSPITAL | 38-59-09.4200N | 077-00-06.9300W |
| TANEYTOWN | EVAPCO | 39-39-13.3580N | 077-11-00.9400W |
| TOWSON | BLACK & DECKER/PARKING LOT 2 | 39-23-56.3840N | 076-35-17.8830W |
| TOWSON | ST JOSEPH HOSPITAL | 39-23-18.0000N | 076-36-36.0000W |
| WESTMINSTER | CARROLL HOSPITAL CENTER | 39-33-31.0000N | 076-59-27.0000W |
| WORTON | NUODEX INC. | 39-15-10.3920N | 076-05-03.8090W |

## 5400 List of Certified Helos/Aircraft Providers

Table 4 Mass Rescue Operation Supplemental Checksheet

## 5500 Fuel/Maintenance Sources

See 5100 for list of Airports with fuel sources.

## 5600 Air Traffic Control Procedures

The FAA and MD Aviation Administration are two sources of information that can be helpful. OPS Section Chief shall ensure air space deconfliction. Consider use of Temporary Flight Restrictions (TFR) if required.