Health and Safety Plan (HASP)

Summary of Washington State Requirements

Purpose: To assist you in developing a comprehensive, written HASP that will meet both federal and state requirements to support a response. State requirements can be found in Chapters 296-824 and 296-843 of the Washington Administrative Code (WAC).

Lessons learned: The primary issues that historically have caused delays in getting a HASP signed off by unified command:

- Not including all potential tactics in your analysis – this means that your plan should not be completed until the tactics process is well under way, or possibly until the Tactics meeting has occurred.

- Not referencing the state regulations and including all required components in the HASP. Plans that only reference 29 CFR 1910.120 will likely not meet Washington State requirements.

- Need to include in the plan your method to actively communicate the contents of the plan to all responders, including those working in the field.

- Order enough safety staff to support the needs of the response, including safety personnel for the field.

- Relying only on an ICS-208 as an incident specific Health and Safety Plan

Perspective: The Incident Safety Officer is responsible for the safety of all personnel supporting the incident, and coordinating supporting organization into one cohesive safety plan. Washington is a “State-Plan” State and has delegated authority for some OSHA responsibilities. Washington has additional requirements for Health and Safety Plans above the OSHA requirements (an ICS-208 does not meet Washington’s requirements).

During the initial response, supporting organizations will all follow their written safety policies and develop job hazard analyses that identified the hazards and protective measures for the work they perform.  Within 24-hours, the Safety Officer will develop a full written incident-specific health and safety plan for every supporting person and organization.  It must cover every activity on the incident.

Safety Plan Review, Acknowledgement and Distribution: The HASP must be reviewed and signed by every person working on the incident. Each person must be able to identify:

- The tasks they will be performing.

- The hazards of their work.

- PELs and monitoring.

- The training they will need to perform their work safely.

- How to protect themselves from the hazards.

- How to decontaminate themselves and their equipment after working.

- How and who to report unsafe conditions.

Every person supporting the incident must review and acknowledge the plan.  The plan must be updated as the response progresses to include any new activities and hazards that develop.  Additionally, daily team safety meetings that are documented – more frequently if new hazards are identified

The safety officer should be able to identify factors that can evaluate effectiveness of the plan.  For example, whether safety staff are conducting inspections at worksites for PPE, decontamination, safe work practices, worker acknowledgement of the plan and daily safety briefings, and tracking that controls identified in the plan and being implemented.

What Every Work Should Know: Each worker should be able to identify all of the following before beginning an assignment on the incident:

- What is your task?

- What are your hazards?

- Signs and symptoms of exposure?

- How do you protect yourself?

- What PPE do you use?

- How do you decontaminate?

Some example tasks include:

- On-water skimming

- Vacuum truck recovery in Division A

- GRP boom tending

- Air monitoring

- SCAT

- Beach clean-up

- Aerial Observation

- Waste management

- Wildlife rescue/rehabilitation

Summary of Washington Regulator Requirements:

You must:  Develop and maintain a written site-specific HASP, WAC 296-843-12005

Health and Safety Plan (HASP)

Reference:  If your overall program required under WAC 296-800-140, Accident Prevention Program (APP), meets requirements of this chapter, you don't need to duplicate those portions of your APP in the site-specific health and safety plan (HASP).  However, if you do have an applicable APP, the safety plan should refer to the APP and it should be attached.

You must:  Develop a written HASP for each hazardous waste site, before beginning hazardous waste operations, that includes at least the following:

Hazard analysis:

- Identification and evaluation of on-site safety and health hazards

- A safety and health risk (hazard) analysis for each site task and operation that is identified in the comprehensive work plan.

Organization chart:

- An organizational structure that reflects current site operations, including the following:

  - Establish and identify the chain of command

  - Identify the site safety and health supervisor and other personnel responsible for employee safety and health

  - Specify the overall responsibilities of supervisors and employees

  - Include the name and title of the person with responsibility and authority to direct all hazardous waste operations

  - Include a site safety and health supervisor responsible for developing and implementing the HASP and verifying compliance

  - Identify the functions and responsibilities of all personnel needed for hazardous waste operations and emergency response.

  - Identify site specific lines of authority, responsibility, and communication.

Comprehensive work plan:

    - A written comprehensive work plan of tasks, objectives, logistics, and resources for site operations, including the following:

      - Addresses anticipated clean-up activities and normal operating procedures unless that information is already available in another document.  You should attach that document.

      - Defines work tasks and objectives

      - Describes how the work tasks and objectives will be accomplished

      - Establishes the personnel requirements to implement the work plan

      - Provides for implementation of training, briefings, and information as required by WAC 296-843-200.

Site control plan:

    - An up-to-date site control plan before clean-up operations begin to minimize employee exposure to hazardous substances and including the following (unless it’s available in another document):

  - A site map

  - Establish site work zones

  - How the “buddy system” is used

  - The site communications plan, including how employees are alerted during emergencies

  - The site’s standard operating procedures (SOPs) or safe work practices

  - Identification of the nearest medical assistance.

Personal protective equipment:

    - A PPE plan that addresses all of the following:

  - Site hazards and activities

  - Methods to evaluate the effectiveness of the PPE plan

  - Criteria for selecting and fitting PPE, including work duration use limitations of particular PPE, and medical considerations such as temperature extremes and heat stress

  - Training on PPE use

  - Procedures for putting on and taking off PPE

  - PPE inspection procedures prior to, during, and after use

  - Decontamination and disposal of PPE

  - Maintenance and storage of PPE.

Additional elements:

    - A sampling and monitoring plan (see WAC 296-843-130) that includes sampling of drums and containers

    - Site control measures (see WAC 296-843-140)

    - Decontamination procedures (see WAC 296-843-150)

    - Spill containment plans (see WAC 296-843-180, Drum and container handling)

    - Standard operating procedures for sampling, managing, and handling drums and containers (see WAC 296-843-180)

    - Entry procedures for tanks or vaults (see chapter 296-809 WAC, Confined spaces)

    - A training, briefings, and information plan (see WAC 296-843-200)

    - A medical surveillance plan (see WAC 296-843-210), that includes site-specific medical surveillance requirements

    - Sanitation (see WAC 296-155-140

    - Lighting (see WAC 296-800-210)

    - Excavations (see chapter 296-155 WAC, Part N, Excavation, trenching and shoring)

    - Any relationship or interaction between other programs and the site-specific program.

Note:

The emergency response plan required by WAC 296-843-160, Emergency Response for Hazardous Waste Sites, is also included as a separate section in the HASP.

You must:  Keep a copy of your HASP on site.

Reference:

For more information, see WAC 296-843-220, Recordkeeping and information access.